UNIVERSITY OF MINES AND TECHNOLOGY (UMaT)

FACULTY OF COMPUTING AND MATHEMATICAL SCIENCES

DEPARTMENT OF Computer Science and Engineering

STADIUM NETWORK SECURITY ASSESSMENT

A Comprehensive Security Report on Perimeter Identification,

Network Infrastructure, and Human Operational Factors

SUBMITTED BY:

| Sulemana Akunatu Abdul Jalil | — FCM.41.018.245.23 |
| --- | --- |
| Mohammed Abdallah | — FCM.41.018.175.23 |
| Arthur Fosu Samuel | — FCM.41.018.070.23 |
| Narteh Benjamin Kwame Tei | — FCM.41.018.183.23 |
| Osei Michael Nyamekye | — FCM.41.018.218.23 |
| Mushinat Mukaila | — FCM.41.018.179.23 |
| Adu Martin Yeboah | — FCM.41.018.013.23 |
| Ernest Dadzie | — FCM.41.018.118.23 |
| Danso Adobea Josephine | — FCM.41.018.120.23 |

SUBMITTED TO:

Department of Computer Science and Engineering

Date: February 2026

Tarkwa, Ghana

# EXECUTIVE SUMMARY

This report presents a comprehensive network security assessment conducted for a 20,000-capacity national stadium facility. The assessment covers three primary areas: the identification and classification of the physical and logical security perimeter, the management of the wireless network boundary, and the analysis of human operational factors that could compromise security.

The stadium has been divided into ten distinct security zones, each carrying a defined risk level and a corresponding set of access controls. Key vulnerabilities have been identified at high-traffic pedestrian gates, service vehicle checkpoints, the Network Operations Centre (NOC), and through potential social engineering attacks targeting staff.

Recommendations are structured around the three-layer security model: physical deterrence, network containment, and human awareness. All findings and controls described in this report are aligned with internationally recognized standards including ISO 27001, NIST SP 800-53, and CIS Controls v8.

# SECTION 1: PERIMETER IDENTIFICATION

## 1.1 Physical Perimeter Definition

The security perimeter encompasses both visible physical boundaries and invisible logical boundaries. Together, these form a layered defense around the stadium facility.

1.1.1 Visible Physical Boundaries

- A 2.4-metre anti-climb steel fence surrounds the entire outer perimeter of the facility

- K4-rated anti-vehicle concrete bollards are installed at all pedestrian and vehicle entry approaches

- Steel-core reinforced doors protect all ground-floor access points to restricted zones

- Full-height turnstile gates at all spectator entrances allow only one person per scan

- Staffed vehicle checkpoints with barriers are positioned at all three vehicle entry points

1.1.2 Invisible Logical Boundaries

- Radio frequency (RF) signal boundary: all wireless signals must terminate within 15 meters inside the outer fence

- CCTV coverage envelope providing continuous 360-degree monitoring around all perimeter zones

- VLAN network segmentation digitally separating all security zones regardless of physical location

- Geofencing alerts triggered when staff-issued devices are detected outside the designated site boundary

## 1.2 Security Zone Classification

The stadium has been divided into ten security zones (Z1 to Z10), each classified according to the sensitivity of the assets contained within and the level of access required.

| Zone | Name | Authorized Personnel | Risk Level | Network |
| --- | --- | --- | --- | --- |
| Z1 | Public Forecourt | General public | LOW | VLAN 100 |
| Z2 | Ticketed Concourse | Ticket holders, stewards | MEDIUM | VLAN 100 |
| Z3 | Seating Bowl | Ticket holders, stewards | MEDIUM | VLAN 100 |
| Z4 | VIP / Hospitality | VIP guests, hospitality staff | MEDIUM | VLAN 100 |
| Z5 | Media / Press Box | Accredited media, IT staff | HIGH | VLAN 40 |
| Z6 | Operations Control Room | Authorized operations staff | CRITICAL | VLAN 30 |
| Z7 | Server Room / NOC | IT and security staff only | CRITICAL | VLAN 10 |
| Z8 | Playing Field | Teams, officials, authorized staff | HIGH | VLAN 30 |
| Z9 | Perimeter Service Road | Vendors, emergency services | HIGH | VLAN 60 |
| Z10 | Parking Areas | Ticket holders, badged staff | MEDIUM | VLAN 100 |

Table 1: Stadium Security Zone Classification

## 1.3 Entry Points and Access Control Mechanisms

1.3.1 Pedestrian Entry Gates

Eight pedestrian gates (A through H) are distributed around the stadium perimeter:

- Gates A, B, and C (North and South) — Primary gates serving the highest volume of spectators

- Gates D, E, F, and G — Secondary gates for supplementary access

- Gate H — Emergency egress only; not used for standard entry

1.3.2 Vehicle Entry Points

- V1 North — Primary vehicle access for VIP and official vehicles

- V2 South Service Gate — Primary access for all vendor, catering, and delivery vehicles

- V3 Emergency Access West — Secondary access reserved for emergency services

1.3.3 Access Control Technologies

| Zone | Primary Control Method | Secondary Control Method |
| --- | --- | --- |
| Z1 — Public Forecourt | None (open public access) | Uniformed steward patrols |
| Z2 — Ticketed Concourse | Barcode / NFC ticket scanning at turnstile | Visual bag search by stewards |
| Z4 — VIP Hospitality | RFID smart card + PIN keypad | Biometric fingerprint scan |
| Z6 — Control Room | RFID card + biometric fingerprint | Video intercom, guard approval |
| Z7 — Server Room / NOC | Iris scan + PIN + RFID card (3-factor) | Mantrap airlock — single occupancy |

Table 2: Access Control Technologies by Zone

## 1.4 Highest Risk Access Points

The following five access points present the greatest security risk and require the highest concentration of resources and controls:

Gate A — North Main Entrance [CRITICAL]

This is the busiest entry point with throughput of up to 3,500 persons per hour during peak arrival periods. The high volume creates significant risk of ticket fraud, counterfeit credential use, and dangerous crowd surge. Dedicated fraud detection scanners, additional steward deployment, and real-time crowd density monitoring are required.

V2 — South Service Gate [CRITICAL]

All vendor and delivery vehicles enter through this point. The risk of concealment of unauthorized persons or prohibited devices within delivery vehicles is rated critical. All deliveries must be pre-registered 24 hours in advance. Every driver and passenger must present government-issued photo identification. Vehicle undercarriage must be inspected with mirrors or automated scanners.

Z7 — Server Room / NOC [CRITICAL]

If an attacker gains physical access to the NOC, they obtain control over the entire stadium network — including CCTV feeds, public address systems, access control infrastructure, and emergency communications. Three-factor authentication (iris scan + PIN + RFID card), a mantrap airlock, and continuous internal surveillance are mandatory.

Gate G — Media West [HIGH]

Media accreditation credentials are relatively easy to counterfeit. Camera and broadcast equipment cases can be used to conceal unauthorized devices. Media personnel also receive pitch-side access. All media credentials must be pre-verified and all equipment must pass X-ray inspection.

V3 — Emergency Access West [HIGH]

This gate is secured with a key and chain and can be opened by a guard on request. An attacker impersonating emergency services personnel could use social engineering to gain entry. Guards at this post must verify all emergency service callouts with the control room before opening this access point.

## 1.5 Surveillance and Monitoring

A total of 266 cameras are deployed across the stadium. Each camera is classified by its primary security function.

| Location | Qty | Camera Type | Function | Classification |
| --- | --- | --- | --- | --- |
| Entry Gates (A–D) | 24 | Fixed 4K bullet | Entry monitoring | PREVENTIVE |
| Forecourts (all sides) | 16 | PTZ dome 4K | Crowd monitoring | PREVENTIVE |
| Concourse (all levels) | 60 | Fisheye 360° | Full coverage | DETECTIVE |
| Seating Bowl | 20 | PTZ long-range | Crowd detection | DETECTIVE |
| NOC / Server Room | 8 | Fixed 4K internal | Physical security | DETECTIVE |
| Parking Areas | 30 | PTZ + pole-mounted | Vehicle monitoring | CORRECTIVE |
| Emergency Exits | 24 | Sensor-triggered | Door alarm coverage | CORRECTIVE |
| Stairwells | 40 | Fixed 2MP dome | Flow monitoring | DETECTIVE |
| Vehicle Checkpoints | 18 | ANPR + overview | Plate recognition | PREVENTIVE |
| Roof / Plant Access | 12 | Motion-activated | Intrusion detection | DETECTIVE |
| VIP Suite Corridors | 8 | Fixed 4K dome | Access monitoring | DETECTIVE |
| Perimeter Service Road | 6 | PTZ wide-angle | Vehicle and staff checks | DETECTIVE |

Table 3: Camera Deployment and Classification (Total: 266 cameras)

Camera classifications are defined as follows:

- PREVENTIVE: Cameras positioned to deter incidents before they occur through visible presence

- DETECTIVE: Cameras positioned to identify and record incidents as they unfold

- CORRECTIVE: Cameras triggered by events to support post-incident response and recovery

1.5.1 Identified Blind Spots

The following areas present reduced camera visibility and require remediation:

- Under-tier concourse pillars: fisheye cameras to be installed at pillar bases

- Toilet and changing room approach corridors: wide-angle cameras required in all approach corridors (not inside facilities)

- Vehicle undercarriage at entry gates: undercarriage mirror scanners at V1 and V2

- Rooftop plant rooms: motion-activated cameras on all maintenance walkways

- Emergency exit exterior zones: fixed cameras covering a minimum 10-metre radius

1.5.2 Lighting Standards

The following minimum lighting levels have been established to support camera performance and staff safety:

- Public forecourt areas: 100 lux minimum

- Concourse and internal circulation: 200 lux minimum

- Parking areas: 50 lux minimum

- Perimeter service roads: 75 lux minimum

- NOC and server room: 500 lux (controlled environment)

# SECTION 2: NETWORK AND WIRELESS PERIMETER

## 2.1 Network Infrastructure Overview

The stadium's network infrastructure is built on a three-tier Cisco architecture (core, distribution, and access layers), housed primarily within the NOC in Zone 7. The design supports 11 discrete VLANs, each carrying a specific category of network traffic.

| VLAN ID | Name | IP Subnet | Purpose |
| --- | --- | --- | --- |
| 10 | IT Management | 10.0.10.0/24 | NOC and IT staff management access |
| 20 | Staff WiFi | 10.0.20.0/23 | All operational staff wireless access |
| 30 | Operations | 10.0.30.0/24 | Control room and operations systems |
| 40 | Media | 10.0.40.0/24 | Broadcast and press equipment |
| 50 | Point of Sale | 10.0.50.0/24 | Retail and catering POS terminals (PCI-DSS scoped) |
| 60 | Emergency | 10.0.60.0/24 | Emergency services communications |
| 70 | CCTV | 10.0.70.0/24 | All surveillance cameras and NVRs |
| 80 | VoIP | 10.0.80.0/24 | Internal telephony and PA systems |
| 100 | Public WIFI | 192.168.100.0/22 | Fan-facing public hotspot (Internet only) |
| 200 | DMZ | 172.16.200.0/24 | Public-facing servers (ticketing, web) |
| 999 | Blackhole | N/A | All unauthorized/unrecognized device traffic dropped here |

Table 4: VLAN Design Summary

## 2.2 Wireless Signal Boundary Management

Controlling the wireless signal boundary is one of the most technically demanding elements of stadium security, as radio signals are invisible, continuous, and capable of passing through structural barriers.

2.2.1 Signal Containment Methods

- Directional antennas installed on all perimeter-facing access points to direct signals inward

- Transmit Power Control (TPC) managed centrally via Cisco Catalyst Center, reducing output to the minimum required level

- RF-shielding window film applied to all windows in the NOC and VIP suites

- Staff SSIDs are hidden — they do not appear in standard WiFi network scans

- 5 GHz and 6 GHz bands are preferred for staff networks due to their inherently shorter propagation range

- Target signal boundary: -75 dBm at the outer perimeter fence line

- Boundary verified using Ekahau Pro RF survey tools; quarterly audits scheduled

## 2.3 Rogue Access Point Detection and Response

A rogue access point is any unauthorized wireless transmitter connected to the network, or any external device broadcasting a wireless signal that could deceive users. Both types represent a significant threat to stadium security.

2.3.1 Rogue Detection via Infrastructure

- All 144 access points operate in dual-mode: simultaneously serving clients and scanning for unknown or suspicious SSIDs

- Any device broadcasting an SSID identical or similar to a legitimate stadium network immediately triggers a containment response

- Containment method: automatic de-authentication broadcasts are sent to disconnect any clients connected to the rogue device

- All alerts are forwarded to the NOC dashboard for investigation within two minutes of detection

2.3.2 Wired Rogue Device Detection

- Cisco Identity Services Engine (ISE) profiles all devices connected to switch ports

- Any unrecognized or unapproved device connecting to a network port triggers an automatic port shutdown and generates a NOC alert

- All switch ports with no active device assignment are administratively disabled

2.3.3 Public WiFi Security Controls

- Public SSID: Stadium_Guest_WiFi on VLAN 100

- All users must complete captive portal registration and accept acceptable use terms before access is granted

- Bandwidth cap: 10 Mbps download, 5 Mbps upload per device

- Cisco Umbrella DNS filtering prevents access to malicious or prohibited websites

- Client isolation enforced: public users cannot reach any other device on the same network

- Public network is physically isolated from all staff and operational VLANs at the firewall layer

2.3.4 Signal Spillover Risk by Zone

| Zone | Spillover Risk | Cause | Mitigation |
| --- | --- | --- | --- |
| North Forecourt (Z1) | HIGH | Adjacent to public street | Directional antennas + TPC reduction |
| Parking Areas (Z10) | MEDIUM | Open air, distant from core | Pole-mounted APs with inward facing antennas |
| VIP Suite Windows | HIGH | Large glass area facing exterior | RF shielding film on all external windows |
| Rooftop Plant Area | LOW | Elevated, minimal occupancy | Minimal AP presence; motion-triggered only |

Table 5: WiFi Signal Spillover Risk Assessment

# SECTION 3: HUMAN OPERATIONAL FACTORS

## 3.1 Actor Classification and Network Trust Levels

All persons interacting with the stadium site or its network have been classified as one of eleven actor types. Each type is assigned a trust level that determines the scope of their network access.

| Actor Type | Zone Access | VLAN Assignment | Trust Level | Network Policy |
| --- | --- | --- | --- | --- |
| General Spectators | Z1–Z3 | VLAN 100 | UNTRUSTED | Internet only, client isolation |
| VIP Guests | Z1–Z4 | VLAN 100 | LOW TRUST | Internet only, managed bandwidth |
| Stewards | Z1–Z3, Z8 | VLAN 20 | MEDIUM | Event systems only |
| Operations Staff | Z2–Z6, Z9 | VLAN 30 | TRUSTED | Operations network, wired preferred |
| IT / NOC Staff | All zones | VLAN 10 | HIGH TRUST | Full network access |
| Security Staff | All zones | VLAN 20+ | HIGH TRUST | CCTV and comms access |
| Media / Broadcast Crew | Z5, Z8 | VLAN 40 | MEDIUM | Media VLAN, outbound only |
| F&B Vendors | Z2, Z3 | VLAN 50 | LOW TRUST | POS terminals only |
| Contractors | Z9, limited | Isolated | UNTRUSTED | No general network access |
| Emergency Services | All zones | VLAN 60 | HIGH (vetted) | Emergency comms only |
| Delivery Drivers | Z9 only | None | UNTRUSTED | No network access granted |

Table 6: Actor Classification and Trust Levels

## 3.2 Social Engineering Threat Analysis

Social engineering refers to techniques that exploit human psychology rather than technical vulnerabilities to gain unauthorized access to systems, physical spaces, or information. The following six scenarios have been identified as the most credible threats at this facility.

Scenario 1: IT Staff Impersonation [CRITICAL]

An attacker poses as an IT technician claiming to need urgent access to a server room or network cabinet for maintenance purposes. A staff member under time pressure may allow access without proper verification.

Countermeasures: All IT staff must carry dual-form photo identification. Any person requesting access to a restricted technical area must be verified by a call-back to the NOC using the internal directory number — not a number provided by the visitor. The two-person rule requires that no individual enters Zone 7 alone. Pre-event photo briefings are issued to all access control guards.

Scenario 2: Evil Twin WiFi Attack [HIGH]

An attacker positioned outside or near the stadium creates a wireless hotspot using the same name (SSID) as the official stadium WiFi. Spectators who connect to this network have their login credentials, payment details, and browsing activity intercepted.

Countermeasures: The rogue AP detection system operates continuously and triggers automatic containment within minutes of detecting an impersonating SSID. Official WiFi names and QR codes are displayed prominently at all entry gates. All stadium systems enforce HTTPS and certificate validation.

Scenario 3: Tailgating and Piggybacking [HIGH]

An unauthorized individual follows a staff member through a secured door without presenting credentials, taking advantage of social politeness or inattention. This is one of the most common physical access attacks.

Countermeasures: A mantrap airlock at the NOC entry physically prevents more than one person from entering at a time. Anti-passback logic in the access control system prevents the same credential from being used for two consecutive entries. Video analytics monitor for tailgating behavior. Staff are trained to challenge anyone entering without presenting credentials.

Scenario 4: Vendor and Supply Chain Deception [CRITICAL]

A fake or compromised delivery is used to smuggle an attacker, prohibited items, or rogue network hardware onto the site. The attacker may take advantage of the time pressure placed on guards during high-volume delivery periods.

Countermeasures: All deliveries must be pre-registered on the delivery manifest at least 24 hours before arrival. No vehicle not on the confirmed manifest is permitted entry. All drivers and passengers must present government-issued photo identification, which is logged. Large consignments are subject to X-ray inspection. All new networking or technical equipment must pass through IT quarantine before deployment on the network.

Scenario 5: Phishing Attack Targeting Staff [HIGH]

A staff member receives a fraudulent email appearing to come from the IT department, requesting them to verify their login credentials or click a link. The link directs them to a fake login page designed to capture their username and password.

Countermeasures: All accounts require multi-factor authentication (MFA), meaning a captured password alone is insufficient. The email security gateway scans all inbound messages for phishing indicators and known malicious domains. Monthly phishing simulation exercises are conducted. All staff email domains are protected with DMARC, DKIM, and SPF records. Privileged accounts used by IT staff require physical YubiKey hardware tokens as the second authentication factor.

Scenario 6: Insider Threat [HIGH]

A current or former staff member intentionally misuses their legitimate access rights to steal data, sabotage systems, disable cameras, or facilitate the entry of unauthorized persons during a live event.

Countermeasures: Role-based access control (RBAC) ensures every staff member has access only to the systems and data required for their specific role. Cisco Stealthwatch and user entity behaviour analytics (UEBA) monitor network activity and generate alerts for anomalous behaviour such as large data transfers, after-hours access, or unusual login locations. Background checks are conducted for all staff in Zones 6 and 7. Account revocation is completed within one hour of any staff member leaving the organisation.

# SECTION 4: SECURITY POLICY AND STANDARDS ALIGNMENT

## 4.1 Standards and Frameworks Referenced

The security architecture described in this report has been designed in alignment with the following internationally recognized standards and frameworks:

| Standard / Framework | Domain | Relevance to This Assessment |
| --- | --- | --- |
| ISO/IEC 27001:2022 | Information Security Management | Overarching ISMS framework for all controls |
| NIST SP 800-53 Rev. 5 | Federal Security Controls | Access control, incident response, and audit |
| NIST SP 800-116 | Physical Access Control | PIV card and biometric access control design |
| NIST SP 800-153 | Wireless LAN Security | WiFi policy and rogue AP detection guidance |
| CIS Controls v8 | Cyber Hygiene | Controls 1–13 applied across network design |
| PCI-DSS v4.0 | Payment Card Security | VLAN 50 POS isolation and segmentation |
| BS EN 62676 | CCTV Standards | Camera specification and placement guidance |
| GDPR Article 6 | Data Protection | Lawful basis for biometric data processing |
| IEEE 802.11ax (Wi-Fi 6) | Wireless Standards | WPA3-Enterprise for all staff networks |

Table 7: Standards and Framework Alignment

## 4.2 Key Policy Recommendations

4.2.1 Access Control Policy

- All access to zones Z5 through Z7 must require at least two factors of authentication

- The two-person rule must be enforced for all entries to Zone Z7 (NOC and Server Room)

- Access rights must be reviewed quarterly; any account inactive for more than 30 days must be suspended

- All access events must be logged to a tamper-evident audit trail retained for a minimum of 12 months

4.2.2 Network Security Policy

- No personal or unregistered device may be connected to any network port without prior authorisation through Cisco ISE

- All staff wireless connections must use WPA3-Enterprise with certificate-based authentication

- Public WiFi must remain strictly isolated from all operational VLANs at the firewall layer at all times

- All new equipment arriving for network deployment must be quarantined and inspected by IT before installation

4.2.3 Surveillance Policy

- All CCTV footage must be retained for a minimum of 31 days in encrypted, access-controlled storage

- Access to live or recorded CCTV footage is restricted to authorized security and IT personnel only

- Quarterly camera audits must be conducted to verify coverage, confirm no blind spots have developed, and test recording quality

- CCTV processing must comply with GDPR Article 6(1)(e) — processing in the public interest — and notices must be displayed at all entry points

4.2.4 Incident Response Policy

- All security incidents must be reported to the NOC within five minutes of detection

- A documented incident response playbook must be maintained and tested twice per year through tabletop exercises

- Post-event security reviews must be completed within 48 hours of any major event

- Security staff must complete refresher training every six months, including social engineering awareness

## 4.3 Risk Register Summary

| Risk | Category | Likelihood | Impact | Risk Level |
| --- | --- | --- | --- | --- |
| Rogue AP / Evil Twin attack | Network | High | High | CRITICAL |
| IT staff impersonation | Human | Medium | High | CRITICAL |
| Fake vendor / supply chain | Physical | Medium | High | CRITICAL |
| Tailgating at secure doors | Physical | High | Medium | HIGH |
| Phishing targeting staff | Human | High | Medium | HIGH |
| Insider threat from staff | Human | Low | High | HIGH |
| WiFi signal spillover | Network | Medium | Medium | MEDIUM |
| Camera blind spots | Physical | Medium | Medium | MEDIUM |
| Counterfeit ticketing at gates | Physical | High | Low | MEDIUM |

Table 8: Risk Register Summary

# CONCLUSION

This security assessment has evaluated the physical, network, and human security posture of a 20,000-capacity national stadium facility. The assessment reveals that the stadium presents a complex security environment where physical boundaries, wireless infrastructure, and human behaviour all interact and must be managed in a coordinated manner.

Three critical risk areas have been identified. First, the physical perimeter particularly the main pedestrian gates, the South Service Gate, and the NOC requires the highest concentration of layered physical and technical controls. Second, the wireless network perimeter must be actively managed to prevent signal spillage, rogue access point attacks, and unauthorized device connections. Third, human factors particularly social engineering attacks  represent the most significant and most difficult threat to defend against, as they target people rather than technology.

The controls proposed in this report, when implemented in full, provide defense in depth across all three of these areas. No single control is sufficient on its own. The combination of physical deterrents, technical enforcement, and staff training and awareness creates a resilient security posture appropriate for a high-profile, high-capacity public venue.

It is recommended that this assessment be reviewed and updated annually, and following any significant change to the facility, its systems, or its operational model.

# REFERENCES

1.   International Organisation for Standardisation. (2022). ISO/IEC 27001:2022 — Information Security Management Systems. ISO.

2.   National Institute of Standards and Technology. (2020). NIST Special Publication 800-53 Rev. 5: Security and Privacy Controls for Information Systems. NIST.

3.   National Institute of Standards and Technology. (2011). NIST SP 800-153: Guidelines for Securing Wireless Local Area Networks. NIST.

4.   Center for Internet Security. (2021). CIS Controls v8. CIS.

5.   PCI Security Standards Council. (2022). PCI-DSS v4.0. PCI SSC.

6.   British Standards Institution. (2014). BS EN 62676-1-1: Video Surveillance Systems for Use in Security Applications. BSI.

7.   European Parliament. (2018). General Data Protection Regulation (EU) 2016/679 (GDPR). Official Journal of the European Union.

8.   Cisco Systems. (2023). Cisco Catalyst Center Configuration Guide. Cisco.

9.   Ekahau Inc. (2023). Ekahau Pro RF Site Survey Methodology. Ekahau.